OFFICE

KOZPONT_BACKUP_R

hostname KOZPONT_BACKUP_R

int befele

ip helper-address 10.1.10.5

no sh

int gig0/0.999

encapsulation dot1q 999 native

int gig0/0.20

ip helper-address 10.1.10.5

encapsulation dot1q 20

ip address 10.1.20.2 255.255.255.0

standby 20 ip 10.1.20.3

standby 20 priority 110

standby 20 preempt

int gig0/0.30

ip helper-address 10.1.10.5

encapsulation dot1q 30

ip address 10.1.30.2 255.255.255.0

standby 30 ip 10.1.30.3

standby 30 priority 110

standby 30 preempt

int gig0/0.40

ip helper-address 10.1.10.5

encapsulation dot1q 40

ip address 10.1.40.2 255.255.255.0

standby 40 ip 10.1.40.3

standby 40 priority 110

standby 40 preempt

int gig0/0.50

ip helper-address 10.1.10.5

encapsulation dot1q 50

ip address 10.1.50.2 255.255.255.0

standby 50 ip 10.1.50.3

standby 50 priority 110

standby 50 preempt

int gig0/0.60

ip helper-address 10.1.10.5

encapsulation dot1q 60

ip address 10.1.60.2 255.255.255.0

standby 60 ip 10.1.60.3

standby 60 priority 110

standby 60 preempt

int gig0/0.70

ip helper-address 10.1.10.5

encapsulation dot1q 70

ip address 10.1.70.2 255.255.255.0

standby 70 ip 10.1.70.3

standby 70 priority 100

standby 70 preempt

int gig0/0.80

ip helper-address 10.1.10.5

encapsulation dot1q 80

ip address 10.1.80.2 255.255.255.0

standby 80 ip 10.1.80.3

standby 80 priority 100

standby 80 preempt

int gig0/0.10

ip helper-address 10.1.10.5

encapsulation dot1q 10

ip address 10.1.10.2 255.255.255.0

standby 10 ip 10.1.10.3

standby 10 priority 100

standby 10 preempt

int gig0/0.200

ip helper-address 10.1.10.5

encapsulation dot1q 200

ip address 10.1.200.2 255.255.255.0

standby 200 ip 10.1.200.3

standby 200 priority 100

standby 200 preempt

int gig0/0.150

ip helper-address 10.1.10.5

encapsulation dot1q 150

ip address 10.1.150.2 255.255.255.0

standby 150 ip 10.1.150.3

standby 150 priority 100

standby 150 preempt

int kifele

no sh

ip add 185.73.12.41 255.255.255.252

track 1 interface kifele line-protocol

standby 10 track 1 decrement 30

KOZPONT_R

hostname KOZPONT_R

int befele

ip helper-address 10.1.10.5

no sh

int gig0/0.999

encapsulation dot1q 999 native

int gig0/0.20

ip helper-address 10.1.10.5

encapsulation dot1q 20

ip address 10.1.20.1 255.255.255.0

standby 20 ip 10.1.20.3

standby 20 priority 100

standby 20 preempt

int gig0/0.30

ip helper-address 10.1.10.5

encapsulation dot1q 30

ip address 10.1.30.1 255.255.255.0

standby 30 ip 10.1.30.3

standby 30 priority 100

standby 30 preempt

int gig0/0.40

ip helper-address 10.1.10.5

encapsulation dot1q 40

ip address 10.1.40.1 255.255.255.0

standby 40 ip 10.1.40.3

standby 40 priority 100

standby 40 preempt

int gig0/0.50

ip helper-address 10.1.10.5

encapsulation dot1q 50

ip address 10.1.50.1 255.255.255.0

standby 50 ip 10.1.50.3

standby 50 priority 100

standby 50 preempt

int gig0/0.60

ip helper-address 10.1.10.5

encapsulation dot1q 60

ip address 10.1.60.1 255.255.255.0

standby 60 ip 10.1.60.3

standby 60 priority 100

standby 60 preempt

int gig0/0.70

ip helper-address 10.1.10.5

encapsulation dot1q 70

ip address 10.1.70.1 255.255.255.0

standby 70 ip 10.1.70.3

standby 70 priority 110

standby 70 preempt

int gig0/0.80

ip helper-address 10.1.10.5

encapsulation dot1q 80

ip address 10.1.80.1 255.255.255.0

standby 80 ip 10.1.80.3

standby 80 priority 110

standby 80 preempt

int gig0/0.10

ip helper-address 10.1.10.5

encapsulation dot1q 10

ip address 10.1.10.1 255.255.255.0

standby 10 ip 10.1.10.3

standby 10 priority 110

standby 10 preempt

int gig0/0.200

ip helper-address 10.1.10.5

encapsulation dot1q 200

ip address 10.1.200.1 255.255.255.0

standby 200 ip 10.1.200.3

standby 200 priority 110

standby 200 preempt

int gig0/0.150

ip helper-address 10.1.10.5

encapsulation dot1q 150

ip address 10.1.150.1 255.255.255.0

standby 150 ip 10.1.150.3

standby 150 priority 110

standby 150 preempt

int kifele

no sh

ip add 102.54.183.1 255.255.255.252

track 1 interface kifele line-protocol

standby 10 track 1 decrement 30

SZERVER_SW

hostname SZERVER_SW

vlan dot1q tag native

vlan 999

name NATIVE

ip dhcp snooping

ip dhcp snooping vlan 10,20,30,40,50,60,70,80,150,200,999

ip arp inspection

ip arp inspection vlan 10,20,30,40,50,60,70,80,150,200,999

vlan 150

name ADMIN

int vlan 150

ip address 10.1.150.8 255.255.255.0

no sh

ip default gateway 10.1.150.3

vtp domain KOZPONT

vtp mode server

vtp password bundaskeny22

vtp version 2

vlan 20

name RECEPTION

vlan 30

name MANAGEMENT

vlan 40

name HR

vlan 50

name MARKETING

vlan 60

name LOGISTIC

vlan 70

name DEVELOPMENT

vlan 80

name COSTUMER_SERVICE

vlan 10

name IT

vlan 200

name GUEST

spanning-tree mode rapid pvst

spanning-tree vlan 10,20,30,40,50,60,70,80,150,200,999 priority 4096

int range FOLDSZINTFELÉ

sw mode trunk

channel-group 1 mode active

interface port-channel 1

ip dhcp snooping trust

ip arp inspection trust

switchport mode trunk

switchport trunk native vlan 999

switchport trunk allowed vlan 10,20,30,40,50,60,70,80,150,200,999

switchport nonegotiate

int range EMELETFELÉ

sw mode trunk

channel-group 2 mode active

interface port-channel 2

ip dhcp snooping trust

ip arp inspection trust

switchport mode trunk

switchport trunk native vlan 999

switchport trunk allowed vlan 10,20,30,40,50,60,70,80,150,200,999

switchport nonegotiate

int ROUTERFELE

ip dhcp snooping trust

ip arp inspection trust

sw mode trunk

switchport trunk native vlan 999

switchport trunk allowed vlan 10,20,30,40,50,60,70,80,150,200,999

switchport nonegotiate

int range VLAN10FELE

ip dhcp snooping trust

ip arp inspection trust

sw mode acc

sw acc vlan 10

in range fa0/10-25

sw mode acc

sw acc vlan 200

spanning-tree bpduguard enable

spanning-tree portfast

storm-control broadcast level 5.00

storm-control multicast level 10.00

storm-control action shutdown

ip dhcp snooping limit rate 15

ip arp inspection limit rate 15

FOLDSZINT_SW

hostname FOLDSZINT_SW

vlan dot1q tag native

vlan 999

name NATIVE

ip dhcp snooping

ip dhcp snooping vlan 10,20,30,40,50,60,70,80,150,200,999

ip arp inspection

ip arp inspection vlan 10,20,30,40,50,60,70,80,150,200,999

vtp domain KOZPONT

vtp mode client

vtp password bundaskeny22

vtp version 2

spanning-tree mode rapid pvst

spanning-tree vlan 10,20,30,40,50,60,70,80,150,200,999 priority 8192

int vlan 150

ip address 10.1.150.6 255.255.255.0

no sh

ip default gateway 10.1.150.3

int SZERVERFELE

sw mode trunk

channel group 1 mode passive

interface port-channel 1

ip dhcp snooping trust

ip arp inspection trust

switchport mode trunk

switchport trunk native vlan 999

switchport trunk allowed vlan 10,20,30,40,50,60,70,80,150,200,999

switchport nonegotiate

int EMELETFELE

sw mode trunk

channel group 3 mode active

interface port-channel 3

ip dhcp snooping trust

ip arp inspection trust

switchport mode trunk

switchport trunk native vlan 999

switchport trunk allowed vlan 10,20,30,40,50,60,70,80,150,200,999

switchport nonegotiate

in range fa0/10-25

sw mode acc

sw acc vlan 200

spanning-tree bpduguard enable

spanning-tree portfast

storm-control broadcast level 5.00

storm-control multicast level 10.00

storm-control action shutdown

ip dhcp snooping limit rate 15

ip arp inspection limit rate 15

EMELET_SW

hostname EMELET_SW

vlan dot1q tag native

vlan 999

name NATIVE

ip dhcp snooping

ip dhcp snooping vlan 10,20,30,40,50,60,70,80,150,200,999

ip arp inspection

ip arp inspection vlan 10,20,30,40,50,60,70,80,150,200,999

vtp domain KOZPONT

vtp mode client

vtp password bundaskeny22

vtp version 2

spanning-tree mode rapid pvst

spanning-tree vlan 10,20,30,40,50,60,70,80,150,200,999 priority 8192

int vlan 150

ip address 10.1.150.7 255.255.255.0

no sh

ip default gateway 10.1.150.3

int SZERVERFELE

sw mode trunk

channel group 2 mode passive

interface port-channel 2

ip dhcp snooping trust

ip arp inspection trust

switchport mode trunk

switchport trunk native vlan 999

switchport trunk allowed vlan 10,20,30,40,50,60,70,80,150,200,999

switchport nonegotiate

int FOLDSZINTFELE

sw mode trunk

channel group 3 mode passive

interface port-channel 3

ip dhcp snooping trust

ip arp inspection trust

switchport mode trunk

switchport trunk native vlan 999

switchport trunk allowed vlan 10,20,30,40,50,60,70,80,150,200,999

switchport nonegotiate

in range fa0/10-25

sw mode acc

sw acc vlan 200

spanning-tree bpduguard enable

spanning-tree portfast

storm-control broadcast level 5.00

storm-control multicast level 10.00

storm-control action shutdown

ip dhcp snooping limit rate 15

ip arp inspection limit rate 15